KWARA STATE POLYTECHNIC, ILORIN

INSTITUTE OF ENVIRONMENTAL STUDIES

DEPARTMENT OF SURVEYING AND GEO-INFORMATICS

A PROJECT REPORT ON:

UTILITY SURVEYING

OF

ROYAL VALLEY ESTATE

AT KUNLENDE/AKEREBIATA ROAD, ILORIN EAST LOCAL GOVERNMENT AREA KWARA STATE

SUBMITTED BY: BABLOLA OLAMIDE GRACE

MATRICULATION: ND/23/SGI/FT/062

SUPERVISOR IN CHARGE: SURV. DIRAN FELIX & SURV. ADEBANJI

INSTITUTION: INSTITUTE OF ENVIRONMENTAL STUDIES

JUNE, 2025

CERTIFICATE

I, BABLOLA OLAMIDE GRACE with Matric number ND/23/SGI/FT/062 hereby certify that the information contained in this project report were obtained in a result of observations and movements taken by me and the utility Survey was done in accordance to Surveying and regularization and Department instructions.

Signature of student:				_________________________________

Name of student:				_________________________________

Date of completion:				_________________________________

Matric Number: 				ND/23/SGI/FT/062

CERTIFICATION

This is to certify that BABLOLA OLAMIDE GRACE with Matric number ND/23/SGI/FT/062 carried out this project work and has been approved as meeting the requirement for the award of National Diploma (ND) in Surveying and Geo-informatics in the Department of Surveying and Geo-informatics of the Institute of Environmental Studies, Kwara State Polytechnic, Ilorin.

Project supervisor 								________________

SURV. DIRAN FELIX 							Signature and Date

Project Coordinator								________________

SURV. AWOLEYE RAPHAEL S.						Signature and Date

Head of Department								________________

SURV. AMBIMBILA ISAU							Signature and Date

External Moderator								_________________

SURV. J.O. OPALEYE							Signature and Date

DEDICATION

This project is dedicated to Almighty God, the creator of heaven and earth, ancient of days, the I am that a I am, the beginning and the end whose supremacy in the knowledge of everything is absolute. To my parents, Mr. and Mrs. Babalola and to my supervisors Surveyor Diran Felix and Surveyor Banji and my noble HOD SURV. AMBIMBIMBOLA ISAU.

ACKNOWLEDGEMENT

All glory be to God Almighty for the successful completion of my National Diploma (ND) programme. He who made all things and by whom all things exist deserves all the praise for the privilege to be a student of Kwara State Polytechnic.

My sincere appreciation goes to my beloved parents, Mr. & Mrs. Babalola — thank you for your unwavering love and sacrifices. May you continue to enjoy long life and sound health.

To my amazing project supervisor, Surv. Diran Felix, and Surv. Adebanji — I deeply appreciate your guidance and support throughout the course of this project. God bless you both abundantly.

Special thanks to the Head of Department, Surveying and Geo-Informatics, Surv. Abimbola Isaau, for your dedication and leadership. I also extend my gratitude to all the lecturers and staff of the department —SURV. Ayuba, Mr Bello Felix Diran, SURV. Williams Kazeem, SURV. A.O Akindele and also Director of IES SURV. A.G Aremu. Thank you for your efforts in shaping our academic journey.

Lastly, to my incredible project partner — thank you for your tireless efforts, collaboration, and shared vision. This achievement is ours together, and truly, it's only upward from here!

ABSTRACT

This project is based on utility survey of ROYAL VALLEY ESTATE at Kunlende/akerebiata road, Ilorin, Ilorin East Local Government Area, Kwara State. This project has been divided into different chapters, Chapter one of this project gives an introduction about the project topic as a whole. This establish better understanding of the project so as to know what perimeter and detailing entails as well as the scope and aim of the project. Chapter two is the literature review i.e the works of past professionals and projects that had been done in the past as regards this specific topic which were examined in order to help to shed more light on what perimeter and detailing survey is all about. Chapter three of this project is the methodology which describes how the project was carried out using digital instrument e.g Total station and its accessories from the first page to the final stage. Chapter four of this project is Data presentation which consists of all data acquired from site during the project execution. Chapter five comprises of summary, problems encountered, conclusion and recommendations. Reference comprises of list of names and works of prominent authors or professionals whose works on survey as regards the specific topic were used in the course of this project.

TABLE OF CONTENT

Title page………………………………………………………………………………………i

Certificate………………………………………………………………………………………ii

Certification…………………………………………………………………………………….iii

Dedication………………………………………………………………………………………iv

Acknowledgement………………………………………………………………………………v

Abstract………………………………………………………………………………………….vi

Table of content………………………………………………………………………………….vii

CHAPTER ONE

1.1 Introduction……..…………………………………………………………………………….1

1.2 Aim of Project.……………………...…………………………………………………………2

1.3 Scope of Project…………...……………………………………………………………...3

1.4 Personnel..……………………………………………………………………………………..4

1.5 Area of the Project…………………………………………………………………………….5

CHAPTER TWO

2.0 Literature Review…………………………………………………………………………….7

2.2 Historical Development of Utility Surveys…………………………………………………..8

2.3 Importance of Utility Surveying……………………………………………………………..10

2.4. Cost Control and Financial Savings…………………………………………...…………….13

2.5. Improved Engineering Design and Constructability……………..…………………………14

CHAPTER THREE

3.0 	Methodology………………………………………………………………………….17

3.1 	Reconnaissance……………………………………………………………………….17

3.2	Instrument Selection………………………………………………………………….19

3.3 	Instrument Test…………………………………………………...…………………...19

3.4 	Monumentation…………………………………………………………………..……21

3.5 	Control Check…………………………………………………………………………22

3.6 	Data Acquisition……………………………………………………………………….25

CHAPTER FOUR

4.0   	Data Processing…………………………………………………………………………26

4.1	Data Editing……………………………………………………………………………...26

4.2	Data Processing using Autocad………………………………………………………….27

4.3	Data Extraction…………………………………………………………………………..28

4.4  	Back Computation………………………...……………………………………………..28

CHAPTER FIVE

5.0 	Summary, Problems Encounterred, Conclusion, Recommendation……………..……...33

5.1 	Summary…………………………………………………………………………………33

5.2 	Problem Encountered……………………………………………………………………33

List of Figures

Figure 3.1 showing the instrument set up for collimation test

Figure 3.2 Diagram and Dimension of Pillars buried

List of Tables

Table 1.1 Showing the List of Personnels

Table 3.1 	Showing the List of the Coordinate of The Control Pillars Used

Table 3.2 	Showing the Test of Instrument Result

Table 3.3	The Given Coordinate of The Control Points

Table 3.4	The Backward Computation for The Given Controls Pillars

Table 3.5	Show the Observed Coordinate of The Controls Pillars

Table 3.6	Show the Backward Computation for The Observed Control Pillars

Table 3.7	Show the Difference Between the Given Control Points and the Observed

Control Pillars

Table 4.1 	Showing Boundary Coordinates

Table 4.1.1 	Showing Back Computation for The Boundary Coordinate

Table 4.1.2	Showing Area Computation for The Boundary Coordination Using Double

Latitude

CHAPTER ONE

1.0. INTRODUCTION

As a student of Surveying and Geo-Informatics, I have come to appreciate the critical role that accurate spatial data plays in the planning, development, and maintenance of infrastructure. One important aspect of surveying that has become increasingly significant in modern infrastructure development is utility surveying. Utility survey refers to the process of detecting, locating, mapping, and documenting underground and above-ground utility services such as water pipelines, gas lines, electrical cables, telecommunications networks, sewage systems, and drainage structures.

With rapid urbanization and the expansion of civil engineering projects, the need for precise information regarding the location and condition of existing utilities has become paramount. Construction activities that proceed without reliable utility data risk causing accidental strikes to underground services, which can lead to costly repairs, project delays, safety hazards, environmental pollution, and even loss of life. Therefore, utility surveys contribute greatly to the safe execution of construction and excavation works, and form a critical component of pre-construction site assessments.

The growth and rapid expansion of urban areas across the world, including in developing countries such as Nigeria, has led to increasing complexity in the arrangement and management of underground and above-ground utilities. These utilities include essential infrastructure components such as water pipelines, electricity cables, gas lines, telecommunication cables, fiber optic networks, sewer lines, and storm water drainage systems. As cities and towns continue to grow, there is increasing competition for underground space, resulting in the frequent crossing, overlapping, and congestion of multiple utility networks within a limited subsurface area. In this increasingly complex environment, accurate information about the location, depth, and alignment of existing utilities has become indispensable. Many existing utility records are outdated, inaccurate, incomplete, or completely unavailable, especially in developing regions where record-keeping systems may not have been fully developed or strictly enforced over the years. The absence of reliable utility data poses serious risks during construction, excavation, or maintenance activities, often resulting in accidental utility strikes. Such strikes can lead to service interruptions, financial losses, legal disputes, environmental degradation, injuries, and in extreme cases, fatal

During my academic training in Surveying and Geo-Informatics, I have been introduced to various modern instruments and technologies that aid in utility detection and mapping. These include Ground Penetrating Radar (GPR), Electromagnetic Locators (EML), Global Navigation Satellite Systems (GNSS), Total Stations, and Theodolites. Each of these instruments plays a unique role in detecting different types of utilities based on their physical properties and depth. The data collected from these instruments is processed using specialized software such as AutoCAD, ArcGIS, QGIS, and other GIS applications, which allow for accurate mapping and database creation.

This experience has significantly enhanced my understanding of the complexities and technical requirements involved in utility surveys, bridging the gap between theoretical knowledge and real-world application.

1.2 AIM OF THE PROJECT

The aim of this project is to carry out a comprehensive utility survey to accurately detect, locate, map, and document both underground and above-ground utility infrastructures within the selected study area. The project is designed to equip me, as a student of Surveying and Geo-Informatics, with hands-on experience in the practical application of advanced surveying techniques, including Ground Penetrating Radar (GPR), Electromagnetic Locators (EML), Global Navigation Satellite System (GNSS), Total Station, and Geographic Information Systems (GIS), in the detection and mapping of utility networks.

This project also aims to develop my capacity in integrating field-collected data with digital geospatial processing and mapping software, enabling the production of precise and up-to-date utility maps. Such maps are vital for supporting safe construction practices, preventing utility conflicts, and guiding effective infrastructure development and management. Additionally, the project seeks to contribute to the creation of a georeferenced spatial database of utility services that can serve as a valuable resource for planners, engineers, utility companies, and government agencies in decision-making processes related to urban development, risk management, and maintenance planning.

Furthermore, the project aims to foster a deeper understanding of the challenges and complexities involved in utility surveying, including data acquisition in congested urban environments, dealing with incomplete utility records, and addressing safety considerations during survey operations. By undertaking this project, I hope to develop both technical competence and problem-solving skills that are essential for addressing real-world challenges in the field of surveying and geoinformatics.

1.3 SCOPE OF THE PROJECT

The project focuses on the detection, positioning, mapping, and documentation of existing underground and above-ground utility services within the selected study area, Some of the scopes includes;

1. Reconnaissance: Conducting preliminary site inspection and investigation to gather relevant information about the project area, identify possible challenges, determine accessibility, observe existing utility features, and plan the fieldwork operations accordingly.

2.Utility Detection: Utilizing modern survey equipment such as Ground Penetrating Radar (GPR), Electromagnetic Locators (EML), anf other detection instruments to locate buried utility lines such as water pipes, electrical cables, gas lines, telecommunication cables, sewage, and drainage systems.

3. Positioning and Georeferencing: Employing Global Navigation Satellite Systems (GNSS) receivers and Total Stations to determine the horizontal and vertical positions of detected utilities with high accuracy.

4. Data Collection: Systematically recording positional and descriptive data of identified utilities, including depth, material type (where possible), and service category.

5. Data Processing and Analysis: Processing the collected field data using specialized software such as AutoCAD, ArcGIS, or QGIS to generate accurate utility maps.

6. Utility Mapping: Producing detailed, scaled, and georeferenced utility maps that clearly show the spatial distribution of underground and surface utilities within the project area.

7. Database Development: Creating a digital database of utility information that can serve as a reference for future construction, development, maintenance, and management.

8. Safety and Risk Mitigation: Identifying potential conflicts between proposed construction activities and existing utilities to minimize the risks of accidental utility strikes during excavation or development projects.

9. Documentation and Reporting: Compiling comprehensive reports that document the procedures, findings, challenges encountered, and recommendations for future utility management.

1.4 PERSONNEL

| S/N | NAME | MATRIC NUMBER | ROLE |
| --- | --- | --- | --- |
| 1 | BABALOLA OLAMIDE GRACE | ND/23/SGI/FT/062 | AUTHOR |
| 2 | MUHAMMED AISHAT | ND/23/SGI/FT/058 | MEMBER |
| 3. | OLAFIMIHAN EMMANUEL . O | ND/23/SGI/FT/083 | MEMBER |
| 4. | MAFTAU OMOWUNMI AISHAT | ND/23/SGI/FT/054 | MEMBER |
| 5 | AWOLUSI TAIWO IREYEMI | ND/23/SGI/FT/066 | MEMBER |
| 6 | OLAWALE AZEEZ .O | ND/23/SGI/FT/055 | MEMBER |
| 7. | RAHMAN ZAINAB OLAMIDE | ND/23/SGI/FT/064 | MEMBER |

Table 1.1 Showing the list of Personnels

1.5 AREA OF THE PROJECT

The project was carried out at Royal Valley Estate location at kunlende/ Akerebiata road Ilorin east local government. The area was chosen due to its active utility network and its relevance for ongoing or proposed construction projects. Accessibility to the site was convenient, allowing for smooth data acquisition.

CHAPTER TWO

LITERATURE REVIEW

2.1 INTRODUCTION

Beyond safety concerns, utility surveys are vital for effective project planning, cost estimation, risk management, and infrastructure asset management. Accurate and up-to-date utility data enables engineers, urban planners, contractors, and government agencies to make informed decisions, avoid unnecessary rework, minimize conflicts between new and existing infrastructure, and efficiently maintain critical services. As emphasized by Rogers et al. (2012), the long-term sustainability of urban development depends heavily on the availability of reliable spatial information about existing subsurface and above-ground utilities.

The importance of utility surveying has increased globally as urbanization accelerates. Accidental utility strikes during construction projects have become a major cause of concern due to their severe consequences, which may include injury or death of workers, environmental hazards, project delays, financial losses, legal liabilities, and widespread public inconvenience. According to Anspach (2004), utility strikes account for thousands of incidents each year worldwide, many of which could have been prevented through proper subsurface utility investigations prior to project commencement.

The field of utility surveying has evolved significantly from its early dependence on outdated paper records and rudimentary field inspections to the modern application of advanced geospatial technologies such as Ground Penetrating Radar (GPR), Electromagnetic Locators (EML), Global Navigation Satellite Systems (GNSS), Geographic Information Systems (GIS), Light Detection and Ranging (LiDAR), and Building Information Modeling (BIM). These technologies have revolutionized the ability of surveyors to detect, map, analyze, and manage complex utility networks with high levels of precision and confidence.

2.2 HISTORICAL DEVELOPMENT OF UTILITY SURVEYS

The problem of locating and mapping utilities is as old as urban infrastructure itself. In the earliest cities, utility networks were built without proper record-keeping, leading to confusion and accidents when new constructions were attempted. According to Perry and Taylor (2002), utilities such as water and sewer lines were often installed haphazardly without systematic planning, leaving many urban areas with undocumented underground networks.

As cities expanded in the mid-20th century, the consequences of this lack of documentation became more severe. Sterling et al. (2009) explained that accidental strikes of utilities became common, resulting in hazardous explosions, loss of service, significant financial costs, and even fatalities. This realization spurred the gradual emergence of utility surveying as a professional practice.

Technological advances have since played a key role in transforming the field. By the late 20th century, the introduction of ground penetrating radar (GPR), electromagnetic locating devices (EML), and global positioning systems (GPS) brought about a new era of non-invasive subsurface detection (Sterling et al., 2016). The integration of GIS platforms further enhanced the capability to manage, analyze, and visualize subsurface utility information effectively.

The problem of locating and mapping utilities is as old as urban infrastructure itself. In the earliest cities, utility networks were built without proper record-keeping, leading to confusion and accidents when new constructions were attempted. According to Perry and Taylor (2002), utilities such as water and sewer lines were often installed haphazardly without systematic planning, leaving many urban areas with undocumented underground networks.

As cities expanded in the mid-20th century, the consequences of this lack of documentation became more severe. Sterling et al. (2009) explained that accidental strikes of utilities became common, resulting in hazardous explosions, loss of service, significant financial costs, and even fatalities. This realization spurred the gradual emergence of utility surveying as a professional practice.

Technological advances have since played a key role in transforming the field. By the late 20th century, the introduction of ground penetrating radar (GPR), electromagnetic locating devices (EML), and global positioning systems (GPS) brought about a new era of non-invasive subsurface detection (Sterling et al., 2016). The integration of GIS platforms further enhanced the capability to manage, analyze, and visualize subsurface utility information effectively.

In Nigeria and many developing countries, the development of utility surveying has not kept pace with global standards due to: Lack of regulatory enforcement

2.Poor record keeping

3.Informal utility installations

4.Limited technical capacity and equipment

Many urban areas suffer from undocumented or poorly mapped utility networks, increasing risks for construction workers, utility companies, and the general public. However, as urbanization accelerates, Nigerian cities are beginning to adopt modern surveying techniques, GNSS-based mapping, and GIS integration to improve utility management. Government agencies like the Kwara State Geographic Information Service (KW-GIS) where this project was conducted are examples of emerging efforts to build more accurate, digitized utility datasets that will eventually support safer and more efficient infrastructure development.

Summary of Historical Development

The historical development of utility surveys reflects humanity's gradual realization of the importance of accurate, systematic, and technologically driven mapping of buried infrastructure. From empirical practices in ancient civilizations to today's high-precision, digital utility mapping systems, the discipline has undergone significant transformation. The continued advancement of technology and regulatory frameworks promises an even more sophisticated future for utility surveying globally — and particularly in Nigeria, where its importance is now more urgent than ever.

2.3 IMPORTANCE OF UTILITY SURVEYING

Utility surveying is an essential and increasingly critical component of modern urban development, civil engineering, infrastructure management, and environmental protection. The accurate detection, mapping, and documentation of both underground and above-ground utilities provide numerous benefits across several sectors. Its importance is amplified as cities grow denser, utility networks become more congested, and the need for reliable and uninterrupted services intensifies.

Utility surveying is no longer just an optional part of project planning; rather, it forms a vital phase that directly influences safety, cost, project efficiency, environmental sustainability, asset management, and national economic development. The following sub-sections will elaborate on these key aspects of utility surveying’s importance in extensive detail.

2.3.1 DISASTER PREPAREDNESS AND EMERGENCY RESPONSE

Another important aspect of utility surveying is its role in disaster preparedness and emergency management. In the event of natural disasters such as floods, earthquakes, landslides, and hurricanes, utility networks are often among the most vulnerable components of infrastructure systems. Accurate utility data is essential for:

1. Quickly assessing the extent of damage to buried assets.

2.Identifying broken pipelines or exposed cables that could cause secondary hazards.

3. Guiding emergency responders to shut off critical lines (e.g., gas, electricity, water) to prevent fires, explosions, and contamination.

4. Planning temporary rerouting of utility services to affected areas.

When utility data is incomplete or outdated, disaster response operations are delayed, and rescue workers face increased risks. Therefore, utility surveying directly contributes to community resilience and disaster risk reduction.

2.3.2 ENVIRONMENTAL PROTECTION

Utility surveying also plays a vital role in environmental protection. Many underground utilities carry hazardous materials such as natural gas, petroleum products, industrial waste, and sewage. Accidental strikes or leaks can result in: Soil and groundwater contamination.

2. Air pollution (in case of gas leaks).

3. River and marine pollution from ruptured sewer lines or pipelines.

4. Harm to vegetation, wildlife, and ecosystems.

By accurately mapping these utilities, surveyors can help prevent ecological disasters and support sustainable development. Additionally, utility surveys assist environmental agencies in monitoring existing installations for compliance with environmental regulations.

2.3.3. Contribution to National Development Goals

In countries like Nigeria and across Africa, where rapid urbanization and infrastructure deficits are among the major development challenges, utility surveying plays a key enabling role in:

1. Reducing infrastructure project costs.

2. Improving project execution speed.

3. Preventing conflicts between government agencies and private utility providers.

4. Supporting major national development programs in energy, transportation, housing, water supply, and telecommunications.

When integrated into national development strategies, comprehensive utility data becomes a national asset that supports economic growth, poverty reduction, and improved living standards.

2.4. COST CONTROL AND FINANCIAL SAVINGS

Utility surveying contributes significantly to cost savings by preventing costly mistakes that may arise from hitting or interfering with existing utilities. Unexpected discoveries of undocumented or misplaced utilities during construction often lead to: Expensive work stoppages, Emergency repair costs, Relocation of damaged utilities, Penalties and legal claims from utility providers, Extended project durations, Re-design and engineering changes. Khetarpal and Rajput (2012) argue that the cost of conducting thorough utility surveys during pre-construction phases is minimal compared to the financial consequences of unforeseen utility conflicts. Proper utility mapping reduces uncertainty in project planning, allowing for more accurate budgeting and efficient allocation of resources.

Construction projects that encounter unexpected utility conflicts often suffer from major delays, sometimes extending project timelines by weeks or months. Delays not only affect contractors and developers but may also disrupt public services, traffic flow, business operations, and residents' daily lives. Accurate utility surveys contribute to efficient project scheduling and seamless workflow execution by: Allowing better excavation planning

2. Avoiding surprises during site preparation

3. Coordinating utility relocation or protection in advance

4. Minimizing disruptions to existing services

5. Preventing costly rescheduling or equipment downtime

Well-planned projects supported by comprehensive utility surveys can meet deadlines more reliably, improve contractor performance, and increase public confidence in the project’s management.

2.5. IMPROVED ENGINEERING DESIGN AND CONSTRUCTABILITY

Utility data provides engineers with essential spatial information needed to design infrastructure projects that are compatible with existing utility systems. Utility conflicts often dictate the routing, elevation, and structure of new roads, buildings, pipelines, bridges, and other public works. Rogers et al. (2012) explain that comprehensive utility surveys enable engineers to: Avoid clashes with existing underground services, Optimize foundation layouts to prevent interference, Design efficient routes for new pipelines and cables, Determine appropriate vertical and horizontal clearances, Plan for safe relocation or modification of existing utilities

With accurate utility mapping, engineers can develop more feasible, practical, and sustainable designs, avoiding costly rework during construction.  Accurate utility surveys enable engineers to make fully informed design decisions from the earliest stages of a project. With precise knowledge of existing underground and overhead utility layouts, designers can select optimal foundation locations that avoid interference with subsurface pipes, cables, and conduits, Adjust the alignment, slope, and profile of roads, bridges, tunnels, and railways to minimize utility conflicts. Determine the most efficient and least disruptive placement for new utilities. Identify zones where utility congestion may limit design options and call for rerouting or reconfiguration. The design phase benefits greatly from the certainty provided by detailed utility maps, reducing the likelihood of encountering unforeseen obstacles during construction.

2.6 Contribution to National Development Goals (Expanded)

Utility surveying plays a fundamental role in supporting national development objectives, especially in developing countries like Nigeria where infrastructure development is both a priority and a major challenge. Accurate and reliable utility data is critical for effective policy implementation, efficient allocation of national resources, improved service delivery, poverty reduction, and the attainment of broader socio-economic development goals. The following sub-sections elaborate on how utility surveying directly contributes to national development across multiple sectors:

a) Infrastructure Development and Expansion; One of the core pillars of national development is the provision of critical infrastructure such as: Roads and highways Bridges and flyovers, Water supply and sewerage systems, Power generation and distribution networks, Communication infrastructure (fiber optics, telephone, and internet cables) Airports, railways, and seaports, Industrial and economic zones. Accurate utility surveys provide essential data required for planning, designing, and constructing these infrastructures in a coordinated, efficient, and sustainable manner. Without reliable utility maps, infrastructure projects often face serious delays, cost overruns, and safety hazards due to unexpected encounters with existing underground utilities.

As Nigeria and other developing nations experience rapid urbanization, utility surveying is critical for the orderly growth of cities and towns. Reliable utility data helps: Prevent unplanned and haphazard utility installations. Guide master plans for cities and new residential or commercial developments. Ensure adequate separation between utilities to minimize future conflicts. Integrate utilities into smart city frameworks using Geographic Information Systems (GIS) and Building Information Modelling (BIM). Support long-term sustainability and resilience in urban design. A well-organized utility network improves urban living conditions by ensuring reliable access to water, power, sanitation, and communication — all of which are vital components of national development.

2.6.1 Support for Economic Growth and Industrialization

Economic growth depends heavily on reliable infrastructure, including properly managed utility networks. Industries, businesses, and investors are more likely to commit capital where they are assured of uninterrupted access to: Clean water, Stable electricity, Efficient waste disposal, High-speed data and communication networks.

Utility surveying ensures that utility networks are accurately mapped, safely installed, and efficiently maintained. This reduces disruptions that could affect manufacturing, service industries, commercial hubs, and export-oriented businesses. As such, utility surveying plays an indirect but powerful role in boosting industrialization, improving competitiveness, and generating national income.

CHAPTER THREE

3.0 	METHODOLOGY

This aspect refers to a system of methods and principles, rules used in a field of study to achieve the aim and objectives of this project. The modality to successful execute this project was carried out based on the survey rules and regulation, supervisor advice and departmental instructions.

3.1 	RECONNAISSANCE

Reconnaissance survey is a preliminary stage for exploratory investigations conducted to gather initial data concern the choice of Site area of Subject before more detailed research or filed work commenced. Reasons for reconnaissance survey of the project area are the following;

- To know the location of the site.

- To determine the preliminary area/perimeter of the land.

- To know the survey method suitable for the field data acquisition and the instrument need for the it.

Reconnaissance in this project involved preliminary investigations to gather information about the survey area before conducting a detailed survey, reconnaissance of this project was carried out in two way namely as follows.

- Office planning

- Field Reconnaissance

3.1.1 	OFFICE PLANNING

It involved the process adopted in the collection of relevant data and specification about the as-built Surveying with the used of traversing survey method. During the office planning the given coordinate for control point PBIL 100, PBIL 107, and PBIL 110, was obtained and computed as shown in the table 3.1 below. The choice of equipment to be use for project was obtained from the survey store in the department of surveying and Geoinformatic, Kwara State Polytechnic, Ilorin, and it was handled accordingly to the survey rules and regulations and departmental instruction.

TABLE 3.1 Showing the list of the coordinate of the control pillars used

| Stations | Northing (m) | Easting (m) |
| --- | --- | --- |
| PBIL 100 | 939353.130 | 648821.110 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

3.1.2 	FIELD RECONNAISSANCE

Field recce and site visitation was conducted as preliminary survey for investigation of the basic information about a specified project area, location, or subject in the field. It allows the members of the groups to be familiar with the terrain, boundary lines, selection number of station required, Equipment that will be used for the job, pillars number PBIL 100, PBIL 107, and PBIL 110, used as a control for the project and so on. The stations points selected was Inter-visible to one another and finally a reconnaissance diagram was produced which was attached to this project

3.2	INSTRUMENT SELECTION

The equipment used for this project was grouped into two categories;

- Hardware

- Software

3.2.1	Hardware used

- Total Station with its Accessories (KTS-442L)

- Reflector poles

- Field book

- Writing material (pen)

- Survey beacons

- Steel Tape (5m)

- Tripod Stand

3.2.2	Software used

Below are the list of the software used during the project processing;

- AutoCAD 2007

- Notepad

- Microsoft word

- Microsoft Excel

3.3 	INSTRUMENT TEST

The instrument test was carried out before it used for angular observation. The purpose of testing the instrument before used is to know the assurance and maybe other instrument is in good Condition the instrument was set up over station A and it was pointed a Sighted to station B whore a pole Is hold, and the angle was recorded for face left the instrument was turn to farce right on the same point and it was sighted to station B, where a pole is held at the same position and the angle was recorded. The distance between station A and B 1s 10m. The steel tape was hold at 60m an point A and it was extend to point B. the results for the instrument test was shown i to figure 3.3 below and table 3:2 below.

Total station

LINE OF SIGHT

RANGING POLE

20M

STATION  X

TERRAIN

STATION  Y

Figure 3.1 showing the instrument set up for collimation test

Table 3.2 Showing the Test of Instrument Result

| STATION | SIGHT | FACE | HORIZONTAL READING | DEDUCED ANGLE |
| --- | --- | --- | --- | --- |
| A | B | L | 180° 00’ 03” |  |
|  | B | R | 00°00’ 00” | 180° 00’ 03” |

Different in horizontal circle reading: 180° 00’ 03’’

Horizontal collimation error

= 180° 00’ 03’’

= 000° 00 03’’

Collimation error = 00° 00'03"2

Horizontal collimation error = 00° 00’ 01”

Because the collimation  error falls within the allowable 01”. The instrument was in good condition.

3.4 	MONUMENTATION

This is the process in which the beacon is being buried in to ground. On the project site there are same existing  beacons on the boundary lines m, nail with cap (bottle cork) was used as the 	station pegs.

18cm

15cm above the ground

60cm below the ground

18cm

30cm

Nail

Bottle cork

PILLAR							   PEG

FIGURE 3.2	Diagram and Dimension of Pillars buried

3.5 	CONTROL CHECK

The Control check was carried out to know the control point PBIL 100, PBIL 107, and PBIL 110, are still in their positions. The instrument was set on Control FP01107 and the temporary adjustment was carried out i.e centring, levelling, and focusing. Back site was taken to control PBIL 107 and it was recorded.

The fore sight was served to PEG1 and it was recorded Steel tape was used to measure the distance between PBIL 110 and PEGI and it was recorded. The data observed was computed for given coordinate and Observed Coordinate, as shown in the table 3.2 below.

Table 3.3	The Given Coordinate of The Control Points

| STATION | NORTHING (m) | EASTING (m) |
| --- | --- | --- |
| PBI 100 | 939353.130 | 648821.110 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

TABLE 3.4	The Backward Computation for the Given Controls pillars

| From station | Bearing | Distance | ∆N | ∆E | Northing (m) | Easting (m) | To station |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 899353.130 | 688821.110 | PBIL 100 |
| PBIL 100 | 1120 43̍ 17 ̎ | 120.827 | -46.670 | 111.450 | 899306.460 | 688932.560 | PBIL 107 |
| PBIL 107 | 1130 27̍ 09 ̎ | 204.758 | -81.491 | 187.843 | 899224.969 | 689120.403 | PBIL 110 |

PBIL 100 to PBIL 107

BEARING = 1120 43̍ 17 ̎

DISTANCE = 120.827m

PBIL 107 to PBIL 110

BEARING = 1130 27̍ 09 ̎

DISTANCE = 204.758m

TABLE 3.5	Show the Observed Coordinate of the Controls Pillars

| Station | Northing (m) | Easting (m) |
| --- | --- | --- |
| PBIL 100 | 939353.126 | 648821.105 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

TABLE 3.6	Show the Backward Computation for the Observed Control Pillars

| From station | Bearing | Distance | ∆N | ∆E | Northing (m) | Easting (m) | To station |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 899353.126 | 688821.105 | PBIL 100 |
| PBIL 100 | 1120 43̍ 08 ̎ | 120.830 | -46.666 | 111.455 | 899306.460 | 688932.560 | PBIL 107 |
| PBIL 107 | 1130 27̍ 09 ̎ | 204.758 | -81.491 | 187.843 | 899224.969 | 689120.403 | PBIL 110 |

PBIL 100 to PBIL 107

BEARING = 1120 43̍ 08 ̎

DISTANCE = 120.830m

PBIL 107 to PBIL 110

BEARING = 1130 27̍ 09 ̎

DISTANCE = 204.758m

TABLE 3.7	Show the Difference between the Given Control Points and the Observed Control Pillars

| STATION | NORTHING (m) | EASTING (m) |
| --- | --- | --- |
| PBIL 100 | 0.004 | 0.005 |
| PBIL 107 | 0.000 | 0.000 |
| PBIL 110 | 0.000 | 0.000 |

Therefore the difference between the given and observer bearing was 00° 00’ 09” and 00° 00’ 00” and the difference between the given and the observer linear distance was 0.0003m, 0.000m the results obtained fall within the allowable accuracy for both angular and linear observation.

It was concluded the instrument was in good condition and fit to be used for the project.

3.6 	DATA AQUISITION

This was carried out on the field using Total station with its accessories (KTS-442L) was used for coordinating the entire site for both the perimeter and details of the site. The Total station was set on the 	control Pillar PBIL 107, and the temporary adjustment was carried out centering, leveling and 	focusing. The coordinates of the occupied station was input after that the coordinates of the backsight was record as well and the observation was done to backsight pillar and the coordinates was take and check against the given. Having check and found accurate then the perimeter survey commenced for the entire project site.

3.6.1 	DETAILING SURVEY

The detailing survey was done immediately after completely carried out the perimeter traverse to determine the position of the features on the Site was fixed using radiation survey method. During this several stations was established to pick the coordinates of the details and the coordinates was stored on the total station as its observed. This was later downloaded. The details that was picked during the detailing survey was coded in different ways like Building edges as ‘BLD’, Electric Poles ‘EP’, Drainage ‘DR’, Roads Line ‘RD’, Spot Height ‘SP’ and so on.

CHAPTER FOUR

4.0   	DATA PROCESSING

This involved several step to ensure accurate and reliable result. It explains the method used in capturing; processing, downloading, and analyzing the data acquired Data reduction in surveying involves various methods to simplify and summarize the collected data for analysis. These are some common methods of data reduction that involves

- Combining multiple values into a single value. The average value is calculated by summing up all the values and dividing by the number of values.

- Least Square Adjustment (LSA) is a method of data reduction that involves minimizing the sum of the squared errors between observed and calculated values.

- Interpolation is a method of data reduction that involves estimating values between observed data points.

- Spatial data reduction refers to the process of simplifying and reducing the complexity of spatial data, such as geographic information system (GIS) data, while preserving its essential characteristics.

4.1	DATA EDITING

- Open excel

- Click on the desire file

- Set to delimited

- Click on next

- Mack space and comma

- Click

- On next

- Then click on finish

4.2	DATA PROCESSING USING AUTOCAD

The plan produced digitally with AutoCAD software. The following are the procedure used:

STEP 1: Open Notepad

Open Notepad on your computer. You can find it in the start menu (Window)

STEP 2: Create a New File

Click on "file" "New" to create a new file

STEP3: Enter Coordinates

Enter your coordinates in the following format:

X. Y. Data

STEP 4: Save the Fil

Click on "File" > "Save as" and choose a location to save your file. "ALADA Coordinate" txt, 	and select "All files" as the file type.

Importing the Coordinate File Into 2007 AutoCAD

STEP 1: Double click AutoCAD

Go under format

Click on format after that pick your unit

Drawing unit

Type.... Decimal

Precision... 0.000

Angle type Degree/Min/See

Precision Od00

Clockwise.... 노

Unit to scale Meters

Direction control North 270d0

Press "OK"

STEP 2: Create a new drawing

Click on "File" > "Name" to create a new drawing

STEP 3: Import the coordinate file

Click on "Tools" >"Reference" > "Import" and select "ALADA coordinate (.txt)" as the file 	type. Navigate to the location where you saved your coordinate file ("ALADA coordinate .txt") 	and select it.

STEP 4: Configure Import Setting. In the import select "Point" as the object type and choose 	the correct coordinate format (eg. X and Y) click "OK" to import the coordinates.

4.3	DATA EXTRACTION

The data recorded in the field book was picked to compute for the bearing and distance. Forward computation and backward computation after all necessary correction have been applied. Correction like slope correction, tension correction and standardization correction.

4.4  	BACK COMPUTATION

The result get for forward computation was used for Back computation.

The following program was required for back computation

i. Input N2-N1 to get ∆N

ii. Input E2-E1 to get ∆E

iii.Input (∆N2+∆E2 to get distance

iv.Input shift tan ∆E∆Nto get the corrected bearing when there us more station repeat step ¡¡¡ and ii.

Table 4.1 Showing Boundary Coordinates

| Station | Northing (m) | Easting(m) |
| --- | --- | --- |
| FPO1107 | 939366.406 | 648932.560 |
| PL1 | 939240.312 | 648901.750 |
| PL2 | 939309.331 | 648780.785 |
| PL3 | 939292.197 | 648603.139 |
| PL4 | 939438.718 | 648619.716 |
| PL5 | 939467.847 | 648693.558 |
| PL 6 | 939353.126 | 648821.105 |

Table 4.1.1 Showing back computation for the boundary coordinate

| FROM Station | BEARING | DISTANCE | ΔN | ΔE | N |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 89 |
| PBIL 107 | 204 58 29 | 72.971 | -66.148 | 30.810 |  |
| PL1 | 299 42 28 | 139.270 | 69.019 | 120.965 |  |
| PL2 | 295 39 02 | 193.739 | 83.866 | 174.646 |  |
| PL3 | 16 40 37 | 47.520 | 45.521 | 13.637 |  |
| PL4 | 112 42 17 | 79.980 | 30.871 | 73.782 |  |
| PL5 | 113 13 14 | 113.790 | 54.721 | 127.547 |  |
| PL6 | 112 43 59 | 120.870 | 48.666 | 455 |  |

TABLE 4.1.2	showing area computation for the boundary coordination using double latitude

| ΔN | ΔE | product |
| --- | --- | --- |
| 66.148 | × - 30.810 | = 2038.020 |
| 66.148 |  |  |
| 69.019 |  |  |
| -63.277 | × - 120.965 | =7654.302 |
| 69.619 |  |  |
| 83.866 |  |  |
| 89.608 | × -174.646 | -=15649.679 |
| 83.866 |  |  |
| 45.521 |  |  |
| 218.995 | × 13.637 | =2986.435 |
| 45.521 |  |  |
| -30.871 |  |  |
| 233.645 | × 73.782 | =17238.792 |
| -30.871 |  |  |
| -54.721 |  |  |
| 148.653 | × 127.547 | =18883.716 |
| -54.721 |  |  |
| -46.666 |  |  |
| 46.666 | × 111.455 | =5201.159 |
| -46.666 |  |  |
| 00.000 |  |  |

Area =

2038.020+7654.302-15649.679+2986.435+17238.795+18883.716+5201.1592

54002.427-18649.6792

38352.7482

=19176.374=> 1.9 hectares

1.9 Hect meter square

Hectare=19176.37410000

= 1.9176374 Hectare

Linear Accuracy = 1(EL)2-((ED)2TOTALLENGHT

EL=EDN=0.066

ED=EDE=0.072

Total length =793.100

∴LinearAccuracy

1(0.66)2+(0.072)2793.100x = 10.09767793.100

=1:8000

CHAPTER FIVE

5.0 	SUMMARY, PROBLEMS ENCOUNTERRED, CONCLUSION, RECOMMENDATION

5.1 	SUMMARY

5.2 	PROBLEM ENCOUNTERED

- Too much of rainfall which does not allowed site

- Unavailable of the group members due to dean's cup football tournament

- While taking observing there are some object obstructing the view when taking the observation e.g electric pole, building, tree etc.

5.3 	RECOMMENDATION

The department should considered the season and time that the students should be given the terminal project.

5.4 	CONCLUSION

At the end the aim and objective of this project was achieved. It was carried out using third order surveying method this project was carried out in accordance with survey rules and regulation and departmental instruction.

| Name | Code | E | N | h |  | PT260 | SH | 668115.4 | 937791.5 | 317.946 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PT1 | ST17 | 668069.6 | 937900.4 | 324.137 |  | PT261 | SH | 668118 | 937790.7 | 318.041 |
| PT2 | CV | 668071.1 | 937900.4 | 324.553 |  | PT262 | SH | 668117.7 | 937795.6 | 317.967 |
| PT3 | CV | 668072 | 937900.3 | 324.541 |  | PT263 | SH | 668113.5 | 937797.2 | 317.656 |
| PT4 | BLD | 668072.6 | 937900.6 | 324.383 |  | PT264 | SH | 668110.1 | 937800.8 | 317.647 |
| PT5 | BLD | 668066.5 | 937905.8 | 323.724 |  | PT265 | SH | 668110.4 | 937806.9 | 317.571 |
| PT6 | SH | 668069.7 | 937897.9 | 324 |  | PT266 | SH | 668114.1 | 937808.4 | 317.326 |
| PT7 | SH | 668072.6 | 937896.1 | 324.671 |  | PT267 | SH | 668117.8 | 937810 | 317.601 |
| PT8 | SH | 668072.2 | 937891.5 | 324.604 |  | PT268 | SH | 668117.6 | 937814.7 | 317.532 |
| PT9 | SH | 668076.5 | 937892.6 | 324.821 |  | PT269 | SH | 668113.8 | 937816 | 317.212 |
| PT10 | SH | 668083.2 | 937892.3 | 324.831 |  | PT270 | SH | 668109 | 937815.1 | 317.485 |
| PT11 | SH | 668085.6 | 937889.4 | 324.87 |  | PT271 | SH | 668104.5 | 937814.4 | 317.27 |
| PT12 | BLD | 668085.4 | 937886.8 | 324.848 |  | PT272 | SH | 668104.8 | 937819.5 | 317.197 |
| PT13 | SH | 668088.7 | 937888.5 | 324.887 |  | PT273 | SH | 668109.7 | 937818.5 | 317.278 |
| PT14 | SH | 668090.4 | 937893.3 | 324.882 |  | PT274 | SH | 668113.6 | 937819 | 317.192 |
| PT15 | SH | 668095.3 | 937895.1 | 324.957 |  | PT275 | SH | 668117.7 | 937821.2 | 317.334 |
| PT16 | SH | 668095.6 | 937891.4 | 324.958 |  | PT276 | SH | 668114.1 | 937823.2 | 317.152 |
| PT17 | BLD | 668139.4 | 937901.3 | 324.586 |  | PT277 | SH | 668109 | 937824.1 | 317.181 |
| PT18 | CV | 668139.1 | 937901.2 | 316.047 |  | PT278 | SH | 668104.9 | 937822.2 | 317.161 |
| PT19 | CV | 668139.5 | 937900.4 | 316.104 |  | PT279 | SH | 668105 | 937825.4 | 317.107 |
| PT20 | CV | 668137.3 | 937899 | 317.048 |  | PT280 | SH | 668103.5 | 937827.1 | 317.276 |
| PT21 | SH | 668133.7 | 937894.7 | 316.106 |  | PT281 | SH | 668108.1 | 937827.2 | 317.179 |
| PT22 | SH | 668136.7 | 937894.8 | 316.021 |  | PT282 | SH | 668112 | 937827.1 | 317.097 |
| PT23 | BLD | 668137 | 937893.5 | 316.015 |  | PT283 | SH | 668116.6 | 937827.5 | 317.116 |
| PT24 | SH | 668141.1 | 937890.4 | 315.886 |  | PT284 | SH | 668117.8 | 937831.4 | 317.048 |
| PT25 | SH | 668145.9 | 937889.2 | 315.599 |  | PT285 | SH | 668112.9 | 937833.2 | 317.012 |
| PT26 | SH | 668147.8 | 937891.6 | 315.619 |  | PT286 | SH | 668108.8 | 937832.7 | 317.058 |
| PT27 | GT | 668148.7 | 937893.4 | 315.804 |  | PT287 | SH | 668106.3 | 937831.7 | 317.095 |
| PT28 | GT | 668147.9 | 937894 | 315.765 |  | PT288 | SH | 668120.4 | 937780 | 318.343 |
| PT29 | BLD | 668148.9 | 937893.2 | 315.928 |  | PT289 | SH | 668125.9 | 937783 | 318.036 |
| PT30 | SH | 668148.1 | 937886.3 | 315.852 |  | PT290 | SH | 668122.6 | 937785.4 | 318.043 |
| PT31 | SH | 668144.4 | 937882.9 | 315.942 |  | PT291 | SH | 668120.9 | 937789.4 | 318.013 |
| PT32 | SH | 668140.3 | 937879.1 | 316.287 |  | PT292 | BLD | 668120.6 | 937791.6 | 317.908 |
| PT33 | SH | 668144.4 | 937877.2 | 316.236 |  | PT293 | SH | 668125 | 937790.3 | 317.853 |
| PT34 | SH | 668147.4 | 937877.9 | 316.065 |  | PT294 | SH | 668128.5 | 937785.7 | 317.944 |
| PT35 | BLD | 668149.4 | 937879.9 | 315.956 |  | PT295 | SH | 668133 | 937781.5 | 317.952 |
| PT36 | BLD | 668156.1 | 937879.6 | 315.966 |  | PT296 | SH | 668132.9 | 937786.5 | 317.857 |
| PT37 | BLD | 668156 | 937874.1 | 315.966 |  | PT297 | SH | 668132.2 | 937790.7 | 317.807 |
| PT38 | EP | 668150.9 | 937875.2 | 316.093 |  | PT298 | SH | 668137 | 937790.8 | 317.772 |
| PT39 | SH | 668152 | 937878.1 | 315.912 |  | PT299 | SH | 668139.7 | 937786.5 | 317.824 |
| PT40 | BLD | 668146.3 | 937874.1 | 316.297 |  | PT300 | SH | 668140 | 937781.4 | 318.046 |
| PT41 | SH | 668132 | 937896.8 | 316.129 |  | PT301 | SH | 668144.3 | 937781.2 | 318.023 |
| PT42 | BLD | 668128.4 | 937894.7 | 316.378 |  | PT302 | SH | 668145.5 | 937786.4 | 317.727 |
| PT43 | SH | 668124.8 | 937894.3 | 316.451 |  | PT303 | SH | 668146 | 937791.2 | 317.717 |
| PT44 | SH | 668121.3 | 937892 | 316.499 |  | PT304 | SH | 668149.5 | 937790.8 | 317.75 |
| PT45 | SH | 668118.3 | 937894.3 | 316.594 |  | PT305 | SH | 668149.7 | 937786.7 | 317.692 |
| PT46 | SH | 668115.7 | 937888.6 | 316.573 |  | PT306 | SH | 668148.9 | 937781.3 | 317.842 |
| PT47 | BLD | 668113.6 | 937886.9 | 316.529 |  | PT307 | BLD | 668150.4 | 937781.4 | 317.8 |
| PT48 | SH | 668110.4 | 937889.4 | 316.736 |  | PT308 | BLD | 668150 | 937791.5 | 317.803 |
| PT49 | SH | 668108.3 | 937891.8 | 316.596 |  | PT309 | SH | 668113.7 | 937768.9 | 318.789 |
| PT50 | SH | 668104.4 | 937892.2 | 316.593 |  | PT310 | SH | 668113.6 | 937766 | 318.901 |
| PT51 | SH | 668104.2 | 937889.1 | 316.626 |  | PT311 | BLD | 668112.8 | 937763.3 | 319.05 |
| PT52 | SH | 668117.5 | 937886.8 | 316.551 |  | PT312 | SH | 668117.2 | 937761.4 | 318.943 |
| PT53 | SH | 668119.7 | 937883.9 | 316.541 |  | PT313 | SH | 668117.5 | 937764.3 | 318.911 |
| PT54 | SH | 668117.1 | 937883.2 | 316.562 |  | PT314 | SH | 668122.4 | 937765.5 | 318.977 |
| PT55 | SH | 668114.2 | 937882.6 | 316.592 |  | PT315 | SH | 668124.5 | 937762.3 | 318.933 |
| PT56 | BLD | 668113.5 | 937874.4 | 316.659 |  | PT316 | SH | 668122.4 | 937758.6 | 318.969 |
| PT57 | SH | 668115.5 | 937874 | 316.641 |  | PT317 | SH | 668121.7 | 937753.2 | 319.012 |
| PT58 | SH | 668117.6 | 937873.9 | 316.666 |  | PT318 | SH | 668119.4 | 937752.3 | 319.022 |
| PT59 | SH | 668116.9 | 937866.3 | 316.759 |  | PT319 | BLD | 668113 | 937752.1 | 319.283 |
| PT60 | SH | 668114.9 | 937865.7 | 316.81 |  | PT320 | BLD | 668113.1 | 937748.7 | 319.275 |
| PT61 | SH | 668112.2 | 937865.8 | 317.066 |  | PT321 | BLD | 668113.2 | 937730.1 | 319.819 |
| PT62 | BLD | 668111.7 | 937870.2 | 316.664 |  | PT322 | BLD | 668118.1 | 937726.7 | 320.172 |
| PT63 | SH | 668124 | 937868.3 | 316.542 |  | PT323 | BLD | 668118.2 | 937750.2 | 319.117 |
| PT64 | BLD | 668129.2 | 937867.4 | 316.594 |  | PT324 | SH | 668122.5 | 937751.6 | 319.006 |
| PT65 | SH | 668128.1 | 937869.8 | 316.579 |  | PT325 | SH | 668124.5 | 937754.3 | 318.972 |
| PT66 | SH | 668127.9 | 937872.5 | 316.566 |  | PT326 | SH | 668125.4 | 937757 | 318.942 |
| PT67 | SH | 668125 | 937873.8 | 316.569 |  | PT327 | SH | 668126.4 | 937759.8 | 318.902 |
| PT68 | SH | 668123.6 | 937877 | 316.822 |  | PT328 | SH | 668127.7 | 937763.6 | 318.945 |
| PT69 | SH | 668126 | 937879.4 | 316.682 |  | PT329 | SH | 668129.6 | 937766.7 | 319.003 |
| PT70 | SH | 668121.7 | 937869.6 | 316.558 |  | PT330 | SH | 668132.3 | 937766.4 | 319.052 |
| PT71 | SH | 668121.5 | 937862 | 316.817 |  | PT331 | SH | 668132.9 | 937763.9 | 319.098 |
| PT72 | SH | 668123.1 | 937858.4 | 316.857 |  | PT332 | SH | 668133.1 | 937760.8 | 318.995 |
| PT73 | SH | 668118.9 | 937856.2 | 316.91 |  | PT333 | SH | 668133.8 | 937757.1 | 318.991 |
| PT74 | BLD | 668120.4 | 937852 | 316.833 |  | PT334 | SH | 668134 | 937753.4 | 318.981 |
| PT75 | SH | 668116.8 | 937852.9 | 316.833 |  | PT335 | SH | 668134.5 | 937750.7 | 319.07 |
| PT76 | CV | 668119 | 937855.1 | 316.791 |  | PT336 | BLD | 668137.6 | 937750.4 | 319.035 |
| PT77 | CV | 668118.5 | 937855.2 | 316.819 |  | PT337 | SH | 668138.2 | 937752.8 | 319.024 |
| PT78 | SH | 668111.8 | 937851.5 | 317.396 |  | PT338 | BLD | 668140.6 | 937755.3 | 318.91 |
| PT79 | SH | 668115.7 | 937850.6 | 316.866 |  | PT339 | SH | 668137.7 | 937757.7 | 319.018 |
| PT80 | SH | 668118.3 | 937849 | 316.835 |  | PT340 | SH | 668136 | 937762 | 319.013 |
| PT81 | SH | 668118.1 | 937843.1 | 316.939 |  | PT341 | SH | 668138.3 | 937764.7 | 319.044 |
| PT82 | SH | 668113.8 | 937842.7 | 316.91 |  | PT342 | BLD | 668139.4 | 937765.7 | 319.051 |
| PT83 | SH | 668108.1 | 937843.3 | 317.4 |  | PT343 | SH | 668141.4 | 937749.2 | 319.111 |
| PT84 | SH | 668106.1 | 937847.2 | 317.641 |  | PT344 | SH | 668143.4 | 937751 | 319.05 |
| PT85 | SH | 668102.4 | 937849.2 | 317.563 |  | PT345 | SH | 668144.9 | 937753.8 | 318.948 |
| PT86 | SH | 668101.2 | 937843.6 | 317.586 |  | PT346 | SH | 668147.2 | 937751.6 | 318.691 |
| PT87 | SH | 668099.2 | 937840.3 | 317.489 |  | PT347 | SH | 668146.9 | 937747.9 | 319.135 |
| PT88 | SH | 668098.1 | 937836.4 | 317.051 |  | PT348 | SH | 668144.5 | 937745.9 | 319.098 |
| PT89 | SH | 668094.9 | 937829.9 | 317.405 |  | PT349 | SH | 668143.8 | 937741.8 | 319.252 |
| PT90 | SH | 668099 | 937830 | 317.366 |  | PT350 | BLD | 668137.8 | 937739.5 | 319.454 |
| PT91 | SH | 668102.2 | 937835.3 | 317 |  | PT351 | SH | 668140 | 937740.1 | 319.295 |
| PT92 | SH | 668093 | 937842.7 | 317.597 |  | PT352 | SH | 668141.4 | 937742 | 319.263 |
| PT93 | SH | 668090.5 | 937849.3 | 317.02 |  | PT353 | SH | 668145.1 | 937740.6 | 319.475 |
| PT94 | SH | 668088 | 937845.4 | 317.154 |  | PT354 | SH | 668147.6 | 937743.1 | 319.297 |
| PT95 | CV | 668089.1 | 937840.1 | 317.01 |  | PT355 | SH | 668149.8 | 937746.8 | 319.15 |
| PT96 | CV | 668089.5 | 937840.1 | 317.036 |  | PT356 | SH | 668151.4 | 937750.4 | 319.144 |
| PT97 | CV | 668089.7 | 937835.1 | 316.97 |  | PT357 | SH | 668153.9 | 937753.9 | 319.13 |
| PT98 | CV | 668089.1 | 937835.1 | 316.951 |  | PT358 | BLD | 668156.8 | 937753.8 | 319.16 |
| PT99 | SH | 668087.3 | 937837.4 | 317.032 |  | PT359 | BLD | 668155 | 937750.1 | 319.175 |
| PT100 | SH | 668084.9 | 937841.3 | 317.015 |  | PT360 | GT | 668156.7 | 937750.4 | 319.189 |
| PT101 | SH | 668081.1 | 937845.6 | 316.694 |  | PT361 | GT | 668156.4 | 937744 | 319.379 |
| PT102 | BLD | 668082.4 | 937849.5 | 316.774 |  | PT362 | BLD | 668151.3 | 937744.1 | 319.293 |
| PT103 | SH | 668079.2 | 937850.8 | 316.576 |  | PT363 | BLD | 668157.6 | 937720.9 | 319.595 |
| PT104 | SH | 668076.3 | 937851.1 | 316.606 |  | PT364 | BLD | 668075.3 | 937717.9 | 320.555 |
| PT105 | SH | 668076.4 | 937857.2 | 316.522 |  | PT365 | BLD | 668042.2 | 937720.6 | 320.353 |
| PT106 | SH | 668079.6 | 937859 | 316.481 |  | PT366 | BLD | 668036 | 937729.4 | 320.261 |
| PT107 | SH | 668082.3 | 937859.3 | 316.539 |  | PT367 | BLD | 668035.2 | 937744.9 | 319.623 |
| PT108 | SH | 668081.6 | 937865.3 | 316.444 |  | PT368 | BLD | 668011 | 937744.6 | 320.389 |
| PT109 | SH | 668078.7 | 937867 | 316.454 |  | PT369 | BLD | 668000.4 | 937749.6 | 319.55 |
| PT110 | SH | 668075.8 | 937866.3 | 316.351 |  | PT370 | BLD | 667993.6 | 937757.8 | 318.942 |
| PT111 | SH | 668074 | 937869.4 | 316.329 |  | PT371 | BLD | 667980.6 | 937772.8 | 318.59 |
| PT112 | SH | 668078.7 | 937870.4 | 316.547 |  | PT372 | BLD | 667974.3 | 937782.4 | 318.279 |
| PT113 | SH | 668081.1 | 937872.7 | 316.472 |  | PT373 | BLD | 667968.1 | 937789.6 | 318.03 |
| PT114 | SH | 668081.9 | 937877.2 | 316.469 |  | PT374 | RD | 667966.1 | 937793.6 | 317.734 |
| PT115 | SH | 668079 | 937877.3 | 316.472 |  | PT375 | CL | 667964.4 | 937795.8 | 317.839 |
| PT116 | SH | 668076.3 | 937876 | 316.604 |  | PT376 | RD | 667962.9 | 937797.2 | 317.658 |
| PT117 | EP | 668074.3 | 937876.1 | 316.594 |  | PT377 | SH | 667967.3 | 937798.1 | 317.599 |
| PT118 | TM | 668071.8 | 937879 | 316.342 |  | PT378 | SH | 667972.1 | 937795.5 | 317.821 |
| PT119 | TM | 668076.7 | 937878.8 | 316.445 |  | PT379 | SH | 667976.9 | 937798.1 | 317.559 |
| PT120 | TM | 668077.2 | 937885.8 | 316.536 |  | PT380 | SH | 667975.5 | 937802 | 317.229 |
| PT121 | SH | 668079.9 | 937885 | 316.362 |  | PT381 | SH | 667973.8 | 937805.3 | 317.111 |
| PT122 | SH | 668082.8 | 937883.4 | 316.48 |  | PT382 | SH | 667974.4 | 937809.3 | 317.087 |
| PT123 | SH | 668079.8 | 937880.6 | 316.437 |  | PT383 | SH | 667980.3 | 937808.6 | 317.008 |
| PT124 | SH | 668064.8 | 937896.9 | 315.429 |  | PT384 | SH | 667986.3 | 937808.3 | 317.055 |
| PT125 | BLD | 668064.8 | 937885.4 | 315.743 |  | PT385 | SH | 667989 | 937806.7 | 317.471 |
| PT126 | SH | 668069.7 | 937888.7 | 315.93 |  | PT386 | SH | 667991.4 | 937808.2 | 317.321 |
| PT127 | SH | 668069.8 | 937882.8 | 315.817 |  | PT387 | BLD | 667995.8 | 937809.1 | 317.181 |
| PT128 | EP | 668066.9 | 937881.4 | 316.367 |  | PT388 | BLD | 667996.6 | 937819.4 | 317.014 |
| PT129 | BLD | 668073.2 | 937873.9 | 316.28 |  | PT389 | BLD | 667997.1 | 937825.8 | 316.715 |
| PT130 | SH | 668076.5 | 937842.5 | 316.803 |  | PT390 | BLD | 667997.7 | 937836.1 | 316.436 |
| PT131 | SH | 668080.9 | 937837.3 | 316.882 |  | PT391 | SH | 667999.8 | 937837.9 | 316.489 |
| PT132 | SH | 668076.4 | 937833.4 | 317.117 |  | PT392 | SH | 668004.1 | 937837 | 316.697 |
| PT133 | SH | 668079.1 | 937829.3 | 317.088 |  | PT393 | SH | 668006.3 | 937835.7 | 316.836 |
| PT134 | SH | 668084.6 | 937829.3 | 317.095 |  | PT394 | SH | 668012.2 | 937839.5 | 317.101 |
| PT135 | SH | 668084.6 | 937825.2 | 317.166 |  | PT395 | BLD | 668024 | 937834.2 | 316.943 |
| PT136 | SH | 668080.9 | 937824.4 | 317.192 |  | PT396 | EP | 668028.4 | 937833.8 | 316.701 |
| PT137 | EP | 668075.2 | 937830.8 | 317.409 |  | PT397 | SH | 668027.3 | 937830.5 | 316.834 |
| PT138 | SH | 668076.5 | 937824.8 | 317.309 |  | PT398 | SH | 668024.8 | 937827.5 | 316.95 |
| PT139 | SH | 668080.1 | 937820.9 | 317.285 |  | PT399 | SH | 668027.4 | 937825.1 | 316.977 |
| PT140 | SH | 668083.9 | 937818.9 | 317.208 |  | PT400 | SH | 668025.7 | 937818 | 317.088 |
| PT141 | SH | 668084.3 | 937814.6 | 317.38 |  | PT401 | BLD | 668022.6 | 937817.1 | 317.238 |
| PT142 | SH | 668080.3 | 937812.9 | 317.483 |  | PT402 | SH | 668025.6 | 937813.3 | 317.258 |
| PT143 | SH | 668077 | 937811.8 | 317.722 |  | PT403 | SH | 668030.1 | 937809.1 | 317.516 |
| PT144 | EP | 668075.5 | 937812.1 | 317.622 |  | PT404 | BLD | 668031.1 | 937802.8 | 317.66 |
| PT145 | CV | 668075.9 | 937810.5 | 317.869 |  | PT405 | SH | 668027.5 | 937801.2 | 317.556 |
| PT146 | CV | 668076.6 | 937810.4 | 317.877 |  | PT406 | SH | 668031.4 | 937799.3 | 317.671 |
| PT147 | CV | 668076.1 | 937808 | 317.884 |  | PT407 | BLD | 668035 | 937801.7 | 318.075 |
| PT148 | CV | 668076.5 | 937808 | 317.896 |  | PT408 | SH | 668032 | 937800 | 317.586 |
| PT149 | BLD | 668073.4 | 937815.4 | 317.468 |  | PT409 | SH | 668031.4 | 937796 | 317.74 |
| PT150 | BLD | 668073.5 | 937803.4 | 317.824 |  | PT410 | SH | 668033.4 | 937793.9 | 317.725 |
| PT151 | SH | 668077.3 | 937804.7 | 317.737 |  | PT411 | BLD | 668035.2 | 937792.8 | 317.863 |
| PT152 | SH | 668080.4 | 937805.5 | 317.546 |  | PT412 | BLD | 668029.7 | 937792.2 | 318.048 |
| PT153 | SH | 668083.8 | 937807.7 | 317.514 |  | PT413 | BLD | 668029.5 | 937779.7 | 318.482 |
| PT154 | SH | 668084 | 937803.1 | 317.576 |  | PT414 | BLD | 668035.6 | 937839.2 | 316.76 |
| PT155 | SH | 668080.5 | 937801.9 | 317.645 |  | PT415 | BLD | 668030 | 937849.5 | 316.014 |
| PT156 | SH | 668076.8 | 937799.4 | 318.005 |  | PT416 | BLD | 668014.7 | 937849.8 | 315.771 |
| PT157 | SH | 668080.5 | 937797 | 317.729 |  | PT417 | RD | 668001.4 | 937850 | 315.553 |
| PT158 | SH | 668084.2 | 937798.5 | 317.584 |  | PT418 | RD | 667996.5 | 937852.3 | 315.646 |
| PT159 | SH | 668086.6 | 937795.5 | 317.625 |  | PT419 | BLD | 668015.4 | 937870.2 | 315.216 |
| PT160 | SH | 668088.9 | 937794.7 | 317.784 |  | PT420 | RD | 668022.4 | 937889 | 314.617 |
| PT161 | SH | 668087.8 | 937791.7 | 318.04 |  | PT421 | RD | 668020.3 | 937890.7 | 314.467 |
| PT162 | BLD | 668090.2 | 937790.5 | 318.057 |  | PT422 | BLD | 668035.6 | 937896.8 | 314.523 |
| PT163 | SH | 668086 | 937790 | 318.019 |  | PT423 | CV | 668032.5 | 937898.1 | 314.441 |
| PT164 | SH | 668079.9 | 937790.6 | 317.829 |  | PT424 | CV | 668032.1 | 937898.6 | 314.354 |
| PT165 | SH | 668076.8 | 937791.1 | 317.997 |  | PT425 | EP | 668026.8 | 937902.6 | 314.367 |
| PT166 | EP | 668075.9 | 937791.5 | 318.028 |  | PT426 | EP | 668042.4 | 937923.4 | 313.423 |
| PT167 | SH | 668081.3 | 937788.3 | 317.911 |  | PT427 | EP | 668045.7 | 937913.1 | 313.825 |
| PT168 | SH | 668086.2 | 937787 | 318.037 |  | PT428 | CV | 668045.7 | 937915.3 | 314.041 |
| PT169 | SH | 668088.3 | 937782.1 | 318.256 |  | PT429 | CV | 668045.2 | 937915.7 | 314.017 |
| PT170 | SH | 668083.1 | 937782.1 | 318.113 |  | PT430 | RD | 668050.4 | 937924.9 | 313.193 |
| PT171 | SH | 668077.2 | 937783.1 | 318.448 |  | PT431 | RD | 668048 | 937928.1 | 313.163 |
| PT172 | SH | 668077.1 | 937779.4 | 318.482 |  | PT432 | CL | 668049.6 | 937927.2 | 313.142 |
| PT173 | SH | 668080.8 | 937779.2 | 318.382 |  | PT433 | BLD | 668057.8 | 937924.9 | 313.443 |
| PT174 | SH | 668086.1 | 937777 | 318.527 |  | PT434 | BLD | 668139.5 | 937920.4 | 313.76 |
| PT175 | SH | 668088.4 | 937774.7 | 318.351 |  | PT435 | BD | 668156.3 | 937920.7 | 312.957 |
| PT176 | CV | 668076.7 | 937775.7 | 318.733 |  | PT436 | BLD | 668156.4 | 937893.6 | 315.143 |
| PT177 | CV | 668076.4 | 937775.7 | 318.755 |  | PT437 | BLD | 668120.5 | 937803.6 | 317.77 |
| PT178 | SH | 668075.6 | 937773.1 | 318.598 |  | PT438 | SH | 668121.8 | 937806.3 | 317.702 |
| PT179 | BLD | 668073.5 | 937768.6 | 318.708 |  | PT439 | BLD | 668120.3 | 937810 | 317.679 |
| PT180 | SH | 668073.1 | 937767.9 | 318.686 |  | PT440 | SH | 668124.2 | 937808.2 | 317.693 |
| PT181 | SH | 668075.1 | 937765.4 | 318.807 |  | PT441 | SH | 668124.2 | 937805.5 | 317.683 |
| PT182 | BLD | 668076.2 | 937763 | 318.99 |  | PT442 | SH | 668127.7 | 937808.3 | 317.623 |
| PT183 | SH | 668077.8 | 937762.3 | 318.92 |  | PT443 | SH | 668129.9 | 937805.3 | 317.555 |
| PT184 | SH | 668079.4 | 937758.5 | 319.226 |  | PT444 | SH | 668132.3 | 937808.2 | 317.557 |
| PT185 | SH | 668076.9 | 937758.6 | 318.98 |  | PT445 | SH | 668135.2 | 937805 | 317.569 |
| PT186 | SH | 668079.6 | 937754.5 | 319.302 |  | PT446 | SH | 668139.8 | 937808.7 | 317.523 |
| PT187 | SH | 668077.8 | 937753.6 | 319.218 |  | PT447 | SH | 668142.5 | 937806.5 | 317.477 |
| PT188 | SH | 668078.5 | 937746.8 | 319.408 |  | PT448 | SH | 668143.5 | 937805.6 | 317.641 |
| PT189 | SH | 668076.3 | 937744 | 319.51 |  | PT449 | BLD | 668152.8 | 937810.8 | 317.334 |
| PT190 | SH | 668079.7 | 937740.7 | 319.611 |  | PT450 | BLD | 668152.8 | 937803.9 | 317.388 |
| PT191 | SH | 668080.3 | 937743.2 | 319.569 |  | PT451 | BLD | 668120.4 | 937826.9 | 317.189 |
| PT192 | SH | 668078.8 | 937739.9 | 319.631 |  | PT452 | SH | 668123.3 | 937828 | 316.989 |
| PT193 | SH | 668078.8 | 937737.1 | 319.719 |  | PT453 | SH | 668126.8 | 937828.7 | 316.943 |
| PT194 | SH | 668081 | 937736.3 | 319.732 |  | PT454 | SH | 668126.3 | 937832 | 316.931 |
| PT195 | SH | 668078.1 | 937736.2 | 319.726 |  | PT455 | SH | 668123.9 | 937832.8 | 316.937 |
| PT196 | SH | 668074.3 | 937736.5 | 319.73 |  | PT456 | SH | 668121.8 | 937833.5 | 316.965 |
| PT197 | BLD | 668076 | 937739.9 | 319.718 |  | PT457 | SH | 668128 | 937833 | 316.904 |
| PT198 | SH | 668070.5 | 937735 | 319.725 |  | PT458 | SH | 668128.4 | 937836.4 | 316.889 |
| PT199 | SH | 668067.7 | 937732.6 | 319.769 |  | PT459 | SH | 668135.6 | 937832.3 | 316.921 |
| PT200 | SH | 668065.8 | 937735.6 | 319.673 |  | PT460 | SH | 668134.3 | 937834.4 | 316.926 |
| PT201 | SH | 668064.2 | 937739.6 | 319.579 |  | PT461 | SH | 668138.4 | 937829.2 | 316.911 |
| PT202 | SH | 668061 | 937736.3 | 319.632 |  | PT462 | SH | 668141.1 | 937835.5 | 316.628 |
| PT203 | SH | 668060.3 | 937732.6 | 319.705 |  | PT463 | SH | 668120.5 | 937838.7 | 317.002 |
| PT204 | SH | 668057.4 | 937735.8 | 319.652 |  | PT464 | BLD | 668073.3 | 937862.2 | 316.453 |
| PT205 | SH | 668056.3 | 937739.5 | 319.449 |  | PT465 | SH | 668072.4 | 937859 | 316.477 |
| PT206 | SH | 668052.5 | 937736.1 | 319.664 |  | PT466 | SH | 668069.5 | 937857 | 316.409 |
| PT207 | SH | 668050.9 | 937732.6 | 319.757 |  | PT467 | SH | 668065.8 | 937859.2 | 316.319 |
| PT208 | SH | 668048.6 | 937736.2 | 319.724 |  | PT468 | SH | 668064.3 | 937857.5 | 316.229 |
| PT209 | BLD | 668043.3 | 937739.9 | 319.659 |  | PT469 | SH | 668063.9 | 937853.2 | 316.483 |
| PT210 | BLD | 668043.6 | 937731.8 | 319.686 |  | PT470 | SH | 668055.7 | 937859.2 | 316.296 |
| PT211 | BLD | 668075.8 | 937729.8 | 319.966 |  | PT471 | SH | 668050.3 | 937854.7 | 316.365 |
| PT212 | BLD | 668079.9 | 937728.9 | 320.175 |  | PT472 | BLD | 668050.1 | 937852.9 | 316.654 |
| PT213 | BLD | 668079.7 | 937763.4 | 318.77 |  | PT473 | SH | 668056.8 | 937852.8 | 316.47 |
| PT214 | SH | 668080.8 | 937767.6 | 318.711 |  | PT474 | SH | 668063.8 | 937852.2 | 316.438 |
| PT215 | SH | 668083.6 | 937771.3 | 318.618 |  | PT475 | SH | 668067.5 | 937853.8 | 316.473 |
| PT216 | CV | 668088.4 | 937773.9 | 318.388 |  | PT476 | BLD | 668073.3 | 937851.7 | 316.674 |
| PT217 | CV | 668088.4 | 937774.1 | 318.401 |  | PT477 | BLD | 668073.4 | 937838.8 | 317.02 |
| PT218 | CV | 668088.9 | 937774.3 | 318.362 |  | PT478 | SH | 668071.2 | 937834.5 | 316.988 |
| PT219 | CV | 668089.2 | 937774.2 | 318.355 |  | PT479 | SH | 668071.7 | 937829.5 | 317.224 |
| PT220 | CV | 668089.7 | 937774 | 318.348 |  | PT480 | BLD | 668073.4 | 937827.2 | 317.205 |
| PT221 | CV | 668089.7 | 937773.6 | 318.358 |  | PT481 | SH | 668067.6 | 937828.1 | 317.111 |
| PT222 | SH | 668092.5 | 937776.3 | 318.551 |  | PT482 | SH | 668064.6 | 937832.8 | 317.034 |
| PT223 | SH | 668090.5 | 937779.5 | 318.338 |  | PT483 | SH | 668061.8 | 937837.4 | 316.978 |
| PT224 | SH | 668092.9 | 937780.6 | 318.233 |  | PT484 | SH | 668056.2 | 937836.7 | 316.98 |
| PT225 | SH | 668091.7 | 937784.1 | 318.136 |  | PT485 | SH | 668057.4 | 937831 | 317.048 |
| PT226 | SH | 668090.5 | 937787.6 | 318.117 |  | PT486 | SH | 668058.5 | 937827.6 | 317.15 |
| PT227 | SH | 668095.2 | 937788.3 | 318.215 |  | PT487 | SH | 668050.6 | 937828 | 316.962 |
| PT228 | SH | 668098.3 | 937788.8 | 318.206 |  | PT488 | SH | 668049.9 | 937833.6 | 316.97 |
| PT229 | SH | 668098.2 | 937786.1 | 318.204 |  | PT489 | SH | 668049 | 937837 | 316.957 |
| PT230 | SH | 668097.9 | 937782.4 | 318.229 |  | PT490 | BLD | 668043.9 | 937838 | 317.267 |
| PT231 | SH | 668097.4 | 937777.8 | 318.321 |  | PT491 | BLD | 668073.7 | 937792 | 318.073 |
| PT232 | SH | 668097.1 | 937773.1 | 318.529 |  | PT492 | SH | 668073.1 | 937784.7 | 318.318 |
| PT233 | SH | 668100.9 | 937773.3 | 318.58 |  | PT493 | BD | 668073.2 | 937780.3 | 318.439 |
| PT234 | SH | 668102.2 | 937777.1 | 318.319 |  | PT494 | SH | 668069.9 | 937784.4 | 318.355 |
| PT235 | SH | 668101.1 | 937781.3 | 318.285 |  | PT495 | SH | 668064.9 | 937790.3 | 318.076 |
| PT236 | SH | 668100.2 | 937785.2 | 318.239 |  | PT496 | SH | 668062.6 | 937784.4 | 318.262 |
| PT237 | SH | 668102.1 | 937788.4 | 318.071 |  | PT497 | SH | 668061.5 | 937781.1 | 318.253 |
| PT238 | BLD | 668103.3 | 937790.4 | 317.873 |  | PT498 | SH | 668054.5 | 937781.3 | 318.237 |
| PT239 | SH | 668105.1 | 937786.6 | 318.137 |  | PT499 | SH | 668053 | 937785.2 | 318.171 |
| PT240 | SH | 668105.9 | 937783.1 | 318.189 |  | PT500 | SH | 668051.4 | 937789.3 | 318.097 |
| PT241 | SH | 668106.2 | 937778.4 | 318.236 |  | PT501 | SH | 668045 | 937791.3 | 318.148 |
| PT242 | SH | 668106.3 | 937774.5 | 318.368 |  | PT502 | SH | 668044.7 | 937785.9 | 318.238 |
| PT243 | SH | 668106.4 | 937771.7 | 318.607 |  | PT503 | BD | 668043.5 | 937789.4 | 318.204 |
| PT244 | SH | 668109.8 | 937771.4 | 318.611 |  | PT504 | BD | 668046.8 | 937781.7 | 318.191 |
| PT245 | SH | 668114.7 | 937770.8 | 318.727 |  | PT505 | SH | 668050.5 | 937781.3 | 318.192 |
| PT246 | CV | 668118.6 | 937769.6 | 318.768 |  | PT506 | BLD | 668073.6 | 937768.4 | 318.52 |
| PT247 | CV | 668118.6 | 937769.2 | 318.875 |  | PT507 | SH | 668072.8 | 937765.9 | 318.772 |
| PT248 | BLD | 668120.4 | 937768.5 | 318.906 |  | PT508 | SH | 668068.1 | 937767.9 | 318.712 |
| PT249 | SH | 668118 | 937771.4 | 318.745 |  | PT509 | SH | 668064.3 | 937765.9 | 318.846 |
| PT250 | SH | 668114.6 | 937774.6 | 318.45 |  | PT510 | SH | 668061.8 | 937764.8 | 318.757 |
| PT251 | SH | 668112.3 | 937778.2 | 318.273 |  | PT511 | SH | 668059.2 | 937766.6 | 318.779 |
| PT252 | SH | 668116.9 | 937781 | 318.219 |  | PT512 | SH | 668056.8 | 937767.3 | 318.813 |
| PT253 | SH | 668114.7 | 937784 | 318.086 |  | PT513 | SH | 668055.6 | 937765.9 | 317.208 |
| PT254 | SH | 668110.9 | 937786.4 | 318.028 |  | PT514 | SH | 668054.7 | 937765.1 | 317.217 |
| PT255 | SH | 668108.3 | 937788.8 | 318.063 |  | PT515 | SH | 668050.4 | 937765 | 317.161 |
| PT256 | SH | 668106.5 | 937792.3 | 317.922 |  | PT516 | SH | 668048.2 | 937766.1 | 317.281 |
| PT257 | SH | 668104.3 | 937794.3 | 317.593 |  | PT517 | SH | 668047.6 | 937767.1 | 318.799 |
| PT258 | SH | 668106.6 | 937796.8 | 317.885 |  | PT518 | BLD | 668043.7 | 937764.7 | 322.083 |
| PT259 | SH | 668110.6 | 937794.3 | 317.866 |  |  |  |  |  |  |